ATTACHMENT A

Lease No: DTFA  -    - L -

I. SECTION A - GENERAL BUILDING REQUIREMENTS AND SPECIFICATIONS

A1 – Adhesives and Sealants (1/12)

The Lessor shall use adhesives and sealants that contain no formaldehyde or heavy metals.

A2 – Ceilings (10/96)

Must have acoustical treatment with a flame spread of 25 or less and smoke development rating of 50 or less.

A3 – Doors (4/12)

Exterior doors shall be weather tight, equipped with cylinder locks and door checks, automatic door closures and open outward. The FAA will be furnished at least two master keys and two keys for each lock. Interior doors must be solid cored and at least 32 by 80 inches with a minimum opening of 32 inches and be of sturdy construction. Fire doors shall conform to NFPA Standard No. 80. As designated by the FAA, doors shall be equipped with non-removable hinge pins, and locks with 7-pin removable cores. The FAA shall provide cores.

A4 – Floor Load (4/12)

All adjoining floor areas shall be 1) of a common level not varying more than 1/4 inch over a 10-foot, 0-inch horizontal run in accordance with the American Concrete Institute standards, 2) non-slip, and 3) accepted by the Real Estate Contracting Officer (RECO).

Under floor surfaces shall be smooth and level. Office areas shall have a minimum live load capacity of 50 pounds per square foot plus 20 pounds per square foot for moveable partitions. Storage areas shall have a minimum live load capacity of 100 pounds per square foot including moveable partitions. A report showing the floor load capacity, at no cost to the Government, by a registered professional engineer may be required by the RECO.  Calculations and structural drawings may also be required.

A5 – General Health and Safety Standards (4/12)

Local Health, Environmental (OSHA and EPA), and Safety Standards and Building Codes shall be complied with when accomplishing any cleaning, construction, renovation, remodeling, maintenance or testing done in the leased space and areas connected to or integrated with the leased space. Whenever FAA requires work processes or precautions to be provided, the Lessor shall coordinate with the FAA during the work so that proper requirements are met prior, during, and post work.

A6 – HVAC (4/12)

All heating, ventilation and air-conditioning systems that service the leased space must maintain a temperature range of 68-72 degrees Fahrenheit year-round. These temperatures must be maintained during hours of operation throughout the leased premises and service areas regardless of outside temperatures. For purposes of this paragraph A6, compliance with ASHRAE standard ANSI/ ANSI/ASHRAE 62-1999 - Ventilation for Acceptable Indoor Air Quality, will meet FAA’s requirements for indoor air quality.

In order to ensure that there is no degradation of air quality or air flow in the leased premises during the term of the Lease, the Lessor agrees to service the roof and/or ground mounted HVAC units (check for defects, lubricate, make adjustments, change the filters, cleaned and make other necessary service requirements) every 60 days.  Lessor also agrees to service the VAV boxes annually (on or before each lease anniversary date).  Such service will include checking the temperature ranges (refer to section B4), checking all speeds on each fan, cleaning the fans and other components, replacing defective parts and completing other necessary repairs and maintenance.

A7 – Lighting (4/12)

Modern, diffused, energy efficient (T-8 or better) fluorescent fixtures shall be provided at working surfaces that maintain a uniform lighting level of 50 foot candles. Emergency lighting must provide at least 0.5 foot candles of illumination throughout the exit path, including exit access routes, exit stairways, or other routes such as passageways to the outside of the building.

In the event the premises have T-12 (or older) fluorescent fixtures, Lessor agrees to replace them with T-8 or better, with the first 6 months of this lease.

A8 – Painting (4/12)

Prior to occupancy, all surfaces must be newly painted with non-lead based paints in colors acceptable to the FAA. All surfaces must be repainted after working hours at Lessor’s expense at least once every five years. Such repainting includes the moving and returning of the furniture, including dismantling, moving and re-assembling the FAA’s systems furniture, if directed by the FAA, at the Lessor’s expense.  Any existing lead based paint shall be properly maintained and managed per existing regulatory requirements. If there is flaking paint, it would need to be sampled for lead. If containing lead, it would need to be abated prior to occupancy. This could be done either by removal or sealing with an encapsulating material.

A9 – Parking (4/12)

At no additional cost to the FAA, the Lessor shall provide <insert quantity of parking spaces> reserved off-street parking spaces. Compliant accessible parking spaces shall be provided per the ABAAS scoping table. The Lessor shall maintain the parking areas in good repair and provide snow and ice removal, as well as the removal of any obstruction that limits FAA access to its designated spaces.

A10 – Prior Notification (8/02)

A pre-construction meeting shall be held at the facility prior to the commencement of any cleaning, construction, renovation, remodeling, repair, maintenance or testing within the leased premises and areas connected to or integrated with the leased premises. If any items on the checklist are questionable or undone, full resolution of the issues will be expected before the project starts. The pre-construction meeting will be planned, scheduled, and coordinated, with the Government’s supervisor or manager responsible for the facility, at least one week before the execution of the work.

A11 – Restrooms and Drinking Fountains (4/12)

Separate ABAAS compliant toilet facilities for men and women shall be provided on each floor where the FAA leases space. Water closets and urinals shall not be visible when the exterior door is open. Each toilet room shall contain toilet paper dispensers, soap dispensers, paper towel dispensers, waste receptacles; a coin operated sanitary napkin dispenser with receptacle for each toilet in the women’s restroom, disposable toilet seat cover dispensers, a convenience outlet, and hot and cold water for all restrooms. The Lessor shall provide a minimum of two ABAAS accessible chilled drinking fountains with potable water within every 200 feet of travel.

A12-Window and Floor Covering (4/12)

All exterior windows shall be equipped with window covering. Floors will be carpeted with a commercial grade of carpet acceptable (carpet tiles or carpet broadloom) to the FAA. Existing floor and window coverings may be accepted at the discretion of the RECO however; prior to occupancy all carpeting and window coverings shall be cleaned.

At no additional cost to the FAA, the Lessor shall replace carpeting at least every eight (8) years during FAA occupancy or any time during the lease when:

- Backing or underlayment is exposed,

- There are noticeable variations in surface color or texture, and/or

- The condition of the carpet is such that it presents a clear and present danger to pedestrians.

Replacement includes moving and return of furniture including dismantling, moving and re-assembling the FAA’s systems furniture if directed by the FAA

II. SECTION B - SERVICES, UTILITIES, AND MAINTENANCE

B1 – Grounds Maintenance (10/96)

The Lessor shall maintain in good condition landscape plants and lawns. The Lessor shall also remove snow and ice from the entrances, exterior walks and parking areas around the premises, prior to and during the Government’s normal operating hours.

B2 – Landscaping (1/12)

A. Where conditions permit, the site shall be landscaped for low maintenance and water conservation with plants that are either native or well-adapted to local growing conditions.

B. Landscape management practices shall prevent or minimize pollution by:

- Employing practices which avoid or minimize the need for fertilizers and pesticides;

- Prohibiting the use of the 2,4-Dichlorophenoxyacetic Acid (2,4-D) herbicide and organophosphates; and

- Composting/recycling all yard waste.

C. The Lessor shall use landscaping products with recycled content that meets the requirements as required by Environmental Protection Agency's (EPA's) Comprehensive Procurement Guidelines (CPG) for landscaping products. Refer to EPA's CPG web site, WWW.EPA.GOV/CPG.

D. The RECO shall approve the landscaping to be provided in writing.

B3 – Pest Control (4/12)

The Lessor shall exterminate and control pests within the premises within a timely manner as required by the Government. Notice shall be provided to the FAA facility manager and a copy to the RECO before any application of herbicide(s)/pesticide(s) or other chemical pest control. OSHA requirements for Hazard Communication shall apply for the use of hazardous materials used in pest control. Copies of Material Safety Data Sheets (MSDS) for all chemicals applied shall be provided to the FAA before application. Only licensed applicators shall be allowed to apply chemicals. Herbicides/pesticides are not to be applied near the outside air intakes of the building during normal working hours or when the system is in operation.

B4 – Services and Facilities (4/12)

The Lessor shall provide to the Government, as part of the rental consideration and consistent with other parts of Attachment A, the following:

- Heating, air conditioning, and ventilation that provide for the comfortable occupancy of the premises. Temperatures will be thermostatically controlled to maintain a temperature of 68 - 72 degrees Fahrenheit. These temperatures must be maintained throughout the leased premises and service areas, regardless of outside temperatures, during the FAA's normal hours of operation.

- Electrical power to the premises for the operation of lights, communications equipment, and office machines.

- Water (hot and cold) and sewer to leased premises.

- Trash removal for leased premises.

- Bulbs and ballasts

B5 – Utilities Not Provided By The Lessor (4/12)

If the cost of utilities is not included as part of the rental consideration, the Lessor must specify which utilities are excluded. The Lessor shall provide separate meters for utilities to be paid for by FAA. When FAA is to pay for utilities and prior to FAA occupancy, the Lessor will furnish the RECO written confirmation of the meter numbers and location, and certification that these meters will measure FAA usage only. Proration is not permissible. If the cost of utilities for heating, ventilation, and air conditioning is not included as part of the rental considerations, an automatic control system will be provided to assure compliance with heating and air conditioning provisions included in this lease.

III. SECTION C - SAFETY AND FIRE PREVENTION

C1 – Fire and Safety Requirements (4/12)

The facility, its systems and appurtenances must be in compliance with the following fire protection and life safety requirements (FLS):

- State and local building codes in affect at the time of construction or most recent modification, whichever is later.

- State and local fire safety and fire prevention codes.

- National Fire Protection Association, Life Safety Code (NFPA 101), latest edition at the time of lease signing.

- All Occupational Safety & Health Administration requirements including 29 CFR 1910 and 29 CFR 1960, and their associated agreements.

Where compliance with the literal requirements of these standards has not been achieved, the Lessor must document, in writing to the FAA , the specific deviation(s) from these standards and what alternative methods have been employed by the Lessor and accepted by the local jurisdiction (where applicable), as an alternative method of compliance.  Furthermore, where alternative methods of compliance are used in lieu of literal compliance with the FLS requirements listed herein, the approach shall be documented by a Fire Protection Engineer and presented to the FAA for review and concurrence.

The specific list of requirements identified in items C1 (1 through 4 above) includes:

- Federal Aviation Administration Order including:

  - FAA Order 3900.19B, FAA Occupational Safety and Health Program

- FLS Codes Promulgated by the International Code Congress including:

- International Building Code

- International Fire Code

- International Mechanical Code

- National Fire Protection Association Codes & Standards including:

- NFPA 10: Standard for Portable Fire Extinguishers

- NFPA 13: Standard for the Installation of Sprinkler Systems

- NFPA 14: Standard for the Installation of Standpipe Systems

- NFPA 20: Standard for the Installation of Stationary Fire pumps

- NFPA 24: Standard for the Installation of Private Fire Service mains and Their Appurtenances

- NFPA 70: National Electrical Code

- NFPA 72: National Fire Alarm and Signaling Code

- NFPA 75: Standard for the Protection of Information Technology Equipment

- NFPA 90A: Standard for the Installation of Air-Conditioning and Ventilating Systems

- NFPA 92: Standard for Smoke Control Systems

- NFPA 101: Life Safety Code

- NFPA 110: Standard for Emergency and Standby Power Systems

- Occupational Safety & Health Administration regulations including:

- OSHA 29 CFR 1910, Occupational Safety and Health Standards (including Subpart E, Exit Routes and Emergency Planning, & Subpart L, Fire Protection

- OSHA / FAA Agreement titled  FAA Alternate Standard for Fire Safety in Airport Traffic Control Towers, (1998 signature date) (a.k.a. 1960.20)

As provided in this section, all codes, standards, orders and directives refer to the current edition in place at the signing of this lease. If  construction or modifications to the leased premises are undertaken at any time during the term of this Lease, all fire protection and life safety systems must be brought into compliance according to the then-current edition of NFPA and local codes and standards.

The building shall, as required by applicable codes, be equipped with automatic sprinklers which conform to NFPA No. 13; be maintained by the Lessor in accordance with NFPA No. 25 ; be electrically supervised and monitored; and  shall have water-flow alarm switches connected to automatically notify the local fire department or central station (NFPA No. 72). Notification of the fire department or central station shall be accomplished through the building fire alarm system. Regardless of local code requirements, when the leased space (including garage areas under lease by the FAA) is on the 6th floor and above, or below grade, sprinklers are required. Furthermore, leased buildings serving National Airspace System (NAS) air traffic control operations and constructed after June 2012, shall be fully protected with an automatic, electrically supervised sprinkler system designed and installed in accordance with the requirements of NFPA 13.

A manual fire alarm system must be provided, maintained, and tested by the Lessor in accordance with NFPA Standard No. 72 (National Fire Alarm and Signaling Code) under the following circumstances:

- Occupancy of the space is by 10 or more persons, or

- Space is more than 1000 square feet in area, or

- Building is three (3) or more stories in height, or

- Building contains more than 50,000 square feet gross floor area.

The fire alarm system wiring and equipment must be electrically supervised and equipped with an automatic fire department notification system, and must conform to NFPA Standards No. 70 and 72. Engineered smoke control systems, if present, shall be maintained in accordance with the manufacturer’s recommendations and must meet performance requirements of NFPA 92.

Fire-safety mechanisms, equivalent to the requirements stated above in this clause, may be accepted, at the discretion of the RECO, if certified by a Professional Fire Protection Engineer, licensed in the subject property’s state.

Portable fire extinguishers matching the hazards accommodated by the lease must be provided, inspected, and maintained by the Lessor in accordance with NFPA Standard No.10.

When the leased space is located in multi-tenant buildings, the Lessor shall be fully responsible for:

- Development of a building Emergency Action Plan (EAP) and Fire Prevention Plan (FPP)

- Publishing and making copies of the EAP and FPP available to all FAA leased space occupants

- Conducting fire evacuation drills, at least annually

- Conducting review and modification of the EAP and FPP at least annually

- Inviting FAA representation in development, review and modification of the EAP and FPP

The FAA facility manager shall be responsible for the development of the tenant specific EAP and FPP for their lease space and in single tenant buildings.

C2 – Halon (4/12)

Halon must not be used as a fire extinguishing system in any FAA leased space.

C3 – Indoor Air Quality (4/12)

The Lessor shall control contaminants at the source and/or operate the space in such a manner that the indicator levels for carbon monoxide (CO), carbon dioxide (CO2), and formaldehyde (HCHO), are not exceeded. The indicator levels for office area are as follows: CO-9 parts per million (PPM) time weighted average (TWA - 8-hour sample); CO2 - 1,000 PPM (TWA); HCHO - 0.1 PPM (TWA). All indoor air contaminant levels in leased space will be kept below appropriate OSHA regulations or Consensus standards, whichever is stricter. Air quality and facility cleaning will be adequate to prevent the growth of mold, mildew and bacteria. Any visual evidence of these will require immediate sampling and remediation. Moisture/standing water will be controlled to prevent the growth of these.

During working hours, ventilation shall be provided in accordance with the latest edition of ANSI/ASHRAE Standard 62, Ventilation for Acceptable Indoor Air Quality.

The Lessor shall promptly investigate indoor air quality (IAQ) complaints and shall implement controls including alteration of building operating procedures (e.g., adjusting air intakes, adjusting air distribution, cleaning and maintaining HVAC, etc.). The FAA is responsible for addressing IAQ problems resulting from its own activities.

MSDS will be provided to the FAA facility manager as well as a copy sent to the RECO for all cleaning solutions used in the FAA spaces.

C4 – Electrical Safety (4/12)

The Lessor shall ensure electrical safety requirements are met, including grounding, bonding, shielding, control of electrostatic discharge (ESD), and lightning protection requirements, in accordance with:

- 29 CFR 1910, Subpart S, Electrical

- FAA Standard HF-STD-001, Human Factors Design Standard, Chapter 12.4, Electrical Hazards

- DOT Specification FAA-G-2100H, Electronic Equipment, General Requirements

- National Fire Protection Association (NFPA) 70, National Electrical Code

- NFPA 70E, Electrical Safety in the Workplace

- American National Standards Institute/Institute of Electrical and Electronics Engineers (ANSI/IEEE) Standard 1100-2005, Recommended Practice for Powering and Grounding Electrical Equipment

- DOT Standard FAA-STD-019E, Lightning and Surge Protection, Grounding, Bonding and Shielding Requirements for Facilities and Equipment

All hazards associated with electrical equipment shall be marked with labels indicating the hazard, in accordance with FAA-G-2100H, Chapter 3.3.5.5, Markings, Signs, Tags and Symbols.

The Lessor shall ensure that personnel are protected from arc flash hazards, in accordance with NFPA 70E, Electrical Safety in the Workplace and that arc flash warning labels are posted on affected panels, with warning labels meeting the requirements of American National Standards Institute (ANSI) Z53, Series of Standards for Safety Signs and Tags.

C5 – EOSH Requirements (4/12)

The Lessor shall provide space, services, and equipment that comply with the following:

- 29 CFR 1910, Occupational Safety and Health Standards (General Industry)

- FAA Order 3900.19B, FAA Occupational and Health Program

- FAA Standard HF-STD-001, Human Factors Design Standard

- National Fire Protection Association (NFPA) 70, National Electrical Code, and NFPA 70E, Electrical Safety in the Workplace

- Relevant fire codes and building codes

Any equipment used or otherwise provided by the Lessor or Lessor’s contractors or agents that presents a potential safety hazard shall be marked with appropriate warning labels or placards, in accordance with 29 CFR 1910.145, Specifications for Accident Prevention Signs and Tags, FAA HF-STD-001, Human Factors Design Standard, Chapter 12.16, Safety Labels and Placards, and American National Standards Institute (ANSI) Z535.4, Product Safety Signs and Labels.

All equipment described herein shall be designed and installed to be free of mechanical hazards that may injure personnel (sharp projections, unguarded moving parts, etc.), in accordance with FAA-G-2100H, Electronic Equipment, General Requirements, Section 3.3.5.4.

## C6 – Fall Protection (4/12)

The Lessor shall ensure proper fall protection safety systems (railings, toe boards, etc.) are in place for all work areas where FAA personnel are required to perform work at four feet or more above the next lowest level (platforms, catwalks, etc.), in accordance with FAA Order 3900.19B, FAA Occupational Safety and Health Program, Chapter 10, 29 CFR 1910, Occupational Safety and Health Standards (General Industry), and 29 CFR 1926, Safety and Health Regulations for Construction.

## C7 – Hazardous Materials (4/12)

The facility and equipment provided by the Lessor shall minimize the use of lead and mercury, in accordance with FAA Order 1050.10C, Prevention, Control, and Abatement of FAA Environmental Pollution; be free of Class I ozone-depleting substances (ODSs), HCFC-22, HCFC-141b, and HCFC-142b, in accordance with 40 CFR Part 82, Protection of Stratospheric Ozone; and be free of polychlorinated biphenyls (PCBs), in accordance with 40 CFR Part 761.

The Lessor shall ensure that FAA personnel are protected from asbestos hazards, in accordance with 29 CFR 1910.1001, Asbestos, and FAA Order 1050.20A, Airway Facilities Asbestos Control Program.

C8 – Seismic Safety for Equipment (4/12)

The lessor shall ensure that building installed equipment is properly anchored to protect personnel during a seismic event, in accordance with DOT Specification FAA-G-2100H, Electronic Equipment, General Requirements, Section 3.3.5, Personnel Safety and Health, and requirements for the seismic zone in which the facility is located.

C9 – OSHA Requirements (10/96)

The Lessor shall provide space, services, equipment, and conditions that comply with Occupational Safety and Health Administration (OSHA) safety and Health standards (29 CFR 1910 and 1926).

C10 – Radon (10/96)

Radon levels in space leased to the FAA shall not equal or exceed the EPA action level for buildings of four (4) picocuries per liter (pCi/L). If radon levels are found to be at or above 4 pCi/L, the Lessor shall develop and promptly implement a plan of corrective action.

C11 – Refrigerants (8/02)

The Lessor shall identify which refrigerants are used in the HVAC systems in the spaces covered by this lease. The lease should provide for use of refrigerants consistent with EPA and ASHRAE requirements.

C12 – Warranty of Space (4/12)

A. Notwithstanding inspection and acceptance by the Government or any provision concerning the conclusiveness thereof, the Lessor warrants that all space leased to the Government under this contract, spaces above suspended ceilings in the leased space, air plenums elsewhere in the building which service the leased space, engineering spaces in the same ventilation zone as the leased space, public spaces and common use space (e.g., lobbies, hallways) will, at the time of acceptance and during the term of the lease contract, comply with the asbestos containing material (ACM) and polychlorinated biphenyl (PCB) requirements of the Toxic Substance Control Act (TSCA). The RECO shall notify the Lessor in writing of any failure to comply with asbestos requirements, within 30 days after the discovery thereof. All construction by the Lessor is required to comply with the OSHA regulations for Asbestos.

B. The leased premises shall be free of all asbestos-containing material, PCB’s, Radon, and other environmentally hazardous substances. If either ACMs or PCBs are found to be in the leased space, the Government reserves the right to require the Lessor, at no cost to the GOVERNMENT, to take whatever corrective action required by the Toxic Substance Control Act, EPA regulations and state requirements. All facilities constructed prior to 1981 are to have an asbestos building survey conducted by a qualified inspector, including a visual examination and bulk sampling. All ACM survey reports must be sent to the RECO.

C. If the Lessor fails, after receipt of notice, to make correction within <insert the specified period of time>, the Government shall have the right to make the required correction and do any of the following:  a.) charge to the Lessor the costs occasioned to the FAA; b.) withhold the costs from the rent; or c.) terminate the lease agreement. Any such termination of the Lease pursuant to this provision shall be at no cost to FAA.

D. The rights and remedies of the FAA in this clause are not exclusive, and are in addition to any other rights and remedies provided by law and under this contract.

E. Definitions.

- "Acceptance", as used in this clause means the act of an authorized representative of the Government by which the Government assumes for itself, or as an agent of another, the leased premises as ready for occupancy or approves a portion of the premises for occupancy in accordance with the provisions of this lease contract.

- "Correction", as used in this clause, means (i) the removal, encapsulation or enclosure of any friable asbestos materials found in the space leased to the Government, spaces above suspended ceilings in the leased space, air plenums elsewhere in the building which service the leased space, public spaces, engineering spaces in the same ventilation zone as the leased space and common use space (e.g., lobbies, hallways). Following such abatement actions, the Lessor shall adhere to the FAA's required post-asbestos-abatement air monitoring program. (ii) With regard to non-friable asbestos materials in good condition, it means the establishment and execution of a special operations and maintenance program and an abatement plan, approved by the Government, to be implemented from the time the materials are discovered through the remainder of the lease term, and (iii) with regard to PCBs, it involves the removal or retrofitting, in accordance with EPA regulations, of any PCB equipment present in the building.

IV. SECTION D – MISCELLANEOUS

D1 – Adjustment For Vacant Premises (10/96)

If the Government fails to occupy any portion of the leased premises or vacates the premises in whole or in part prior to expiration of the firm term of the lease, the rental rate shall be reduced as follows: The rate shall be reduced by that portion of the costs per square foot of operating expenses not required to maintain the space. Said reduction shall occur after the Government gives 30 days prior notice to the Lessor, and shall continue in effect until the Government occupies the premises or the lease expires or is terminated.

D2 – Condition Report (4/12)

A survey and inspection report of the demised premises will be made as of the effective date of this lease, reflecting the then present condition, and will be signed on behalf of the parties hereto.

D3 – Contracting Officer’s Representative (COR) (10/96)

The Lessor shall address all questions concerning this lease to <insert name of COR> who has been appointed the Contracting Officer's Representative for the administration of this lease, as written.

D4 – Holdover (10/13)

If after the expiration of the lease, the Government shall retain possession of the premises, the lease shall continue in force and effect on a month-to-month basis.  Rent shall be paid in accordance with the terms of the lease, in arrears on a prorated base, at the rate paid during the lease term.  This period shall continue until the Government shall have signed a new lease with the Lessor, acquire the property in fee, or vacated the premises.

D5 – Display Advertising (10/96)

If the leased premises are solely for Government use, no advertising matter shall be constructed on or over the premises, unless authorized by the RECO.

D6 – Erection of Signs (10/96)

The Government shall have the right to erect on or attach to the Lessor's premises such signs as may be required to clearly identify the Government's facility. Said signs so erected will remain the property of the Government and shall be removed from the premises upon termination of the Lease.

D7 – Hold Harmless (10/96)

In accordance with and subject to the conditions, limitations and exceptions set forth in the Federal Tort Claims Act of 1948, as amended (28 USC 2671 et. seq.), hereafter termed "the Act" the Government will be liable to persons damaged by any personal injury, death or injury to or loss of property, which is caused by a negligent or wrongful act or omission of an employee of the Government while acting within the scope of his office or employment under circumstances where a private person would be liable in accordance with the law of the place where the act or omission occurred. The foregoing shall not be deemed to extend the Government's liability beyond that existing under the Act at the time of such act or omission or to preclude the Government from using any defense available in law or equity.

D8 – If Minimum Not Delivered (10/96)

If delivered space contains less than the minimum square footage, the Government may cancel the lease. If such cancellation occurs, the Government may exercise it’s legal rights including charging the Lessor and its surety the increased cost of providing replacement space.

D9 – Installation Of Antennas, Cables And Other Appurtenances (4/12)

The FAA shall have the right to install, operate and maintain antennas, wires and their supporting structures, including any linking wires, connecting cables and conduits atop and within buildings and structures, or at other locations, as deemed necessary by the Government.  The Government will coordinate with the Lessor when installing antennas, cables, and other appurtenances.

D10 – Interest For Late Payments (4/12)

If requested by the Lessor in writing, the Government will pay an interest penalty when payment is not made within (30, 45, 60, 90, etc.) <insert number of days> days of the due date.

Payment shall be considered as being made on the day a check is dated or an electronic funds transfer is made. All days referred to in this clause are calendar days, unless otherwise specified. Rent shall be paid monthly in arrears and will be due on the first workday of each month and only as provided for by the lease.

The interest penalty shall be at the rate established by the Secretary of the Treasury under Section 12 of the Contract Disputes Act of 1978 (41 U.S.C. 611) that is in effect on the day after the due date. This rate is referred to as the "Renegotiation Board Interest Rate," and it is published in the Federal Register semiannually on or about January 1 and July 1. Interest penalties of less than $1.00 need not be paid.

Interest penalties will not be paid on delays due to disagreement between the Government and Lessor over the payment amount, requests for additional information, or for other issues involving contract compliance, or on amounts temporally withheld or retained in accordance with the terms of the contract.

For payments other than rent, the Lessor shall prepare and submit an invoice to the RECO or the RECO’s designated representative for approval not later than 60 days after completion and acceptance of the work. An invoice shall include the following items:

- Name and address of the Lessor.

- Invoice date.

- Lease Number

- Government’s order number or other authorization.

- Description, price, and quantity of work or services delivered.

- Name and address of Lessor official to whom payment is to be sent

D11 – Janitorial Services (4/12)

The Lessor shall maintain the leased premises, including outside areas in a clean condition. The Lessor shall provide the labor, materials, equipment and supervision necessary to ensure such condition.  The Lessor shall provide janitorial services for the leased space, public areas, entrances, and all other common areas and shall provide replacement of supplies.

A.SELECTION OF CLEANING PRODUCTS:

The Lessor shall select, to the maximum extent practicable janitorial cleaning products and equipment that promote environmental stewardship.  At a minimum, the Lessor shall:

- Use products that are packaged ecologically;

- Use products and equipment considered environmentally beneficial and/or recycled products that are phosphate-free, non-corrosive, non-flammable, and fully biodegradable; and;

- Minimize the use of harsh chemicals and the release of irritating fumes.

- Examples of acceptable products may be found at www.gsa.gov/p2products.

B. SELECTION OF PAPER PRODUCTS:

The Lessor shall select paper and paper products (i.e., bathroom tissue and paper towels) with recycled content conforming to EPA’s Comprehensive Procurement Guidance (CPG).

C.SCHEDULE OF CLEANING:

Cleaning shall be performed after the close of the official business day as defined in this lease, unless cleaning during official duty hours is specified as a special requirement.  The required cleaning services, and their frequencies, are set forth below:

1. Daily:

- Sweep floors using chemically treated absorbent or dusting tools (such as DEX or equal).

- Vacuum all carpeted areas, as needed.

- Empty waste baskets and containers; dispose of waste paper, trash, and other extraneous materials.

- Clean toilet rooms, including toilets and soap containers.  Furnish and maintain constant supply of deodorant material and paper products

- Wash all drinking fountains.

- Refill hand sanitizer dispensers in common areas, where applicable

- In the restrooms:

- Clean restroom fixtures and chrome fittings.

- Clean and refill all dispensers (including deodorant material).

- Wet mop restroom floors.

- Sanitize toilets, toilet seats, and urinals.

- Spot wash walls, partitions, and doors.

2. Weekly:

- Dust counters, file cabinets, and telephones, and surfaces of all office furniture, fixtures, and window sills (except desk tops).

- Damp mop, and buff all non-carpeted floors.

- Vacuum all carpeted areas.

3. Monthly:

- Wash waste baskets.

- Damp mop and buff all non-carpeted floors; wax and buff non-carpeted floors.

- Clean or wash walls as needed to present a neat appearance.

- Dust all ledges and flat surfaces within reach.

- Dust and clean all light fixtures.

- Dust and clean all venetian blinds.

- Wash restroom walls, partitions, and doors.

4. Semi-Annually:

- Wash all exterior office windows.

5. Once a Year:

- Strip old wax from all floor space and rewax.  This service is to be performed in conjunction with one of the quarterly cleaning schedules.

- Shampoo all carpeted floors.

Within 60 days after occupancy by the Government, the Lessor shall provide to the RECO with a detailed written schedule of all periodic services and maintenance to be performed other than daily, weekly, or monthly.

D12 – Measurement For Payment (10/96)

The space will be mutually measured upon delivery. Payment will be made on the basis of actual measurement; however, payment will not be made for delivered space, which is in excess of the maximum square footage solicited.

D13 – Non-Restoration (10/96)

The FAA shall have no obligation to restore and/or rehabilitate, either wholly or partially, the premises under this lease. It is further agreed that the FAA may abandon in place any or all of the structures, improvements and/or equipment installed in or located upon said property by the FAA during its tenure. Notice of abandonment will be conveyed to the Lessor in writing.

D14 – Occupancy Permit (8/02)

The premises offered shall have a valid Occupancy Permit, issued by the local jurisdiction, for the intended use of the Government, or  the Offeror shall complete and provide a certified copy of "FAA Safety & Environmental Checklist” form, in lieu of an occupancy permit, at the RECO’s discretion. The leased premises, all accesses to the leased premises, building operations, equipment, services, or utilities furnished by the Lessor, and activities of other occupants, shall be free of safety, health, and fire hazards. When such hazards are detected, they must be promptly corrected at the Lessor's expense.

D15 – Operating Costs Escalator (10/96)

Beginning with the second year of the lease and each year after, Government shall pay adjusted rent for changes in costs for cleaning services, supplies, materials, maintenance, trash removal, landscaping, water, sewer charges, heating, electricity, and certain administrative expenses attributable to occupancy.

The amount shall be determined by multiplying the total first year's estimated costs of these items, as negotiated and established prior to the lease award, by the percent of change in the cost of living index from the base figure. The base figure shall be the index figure published for the month prior to the lease commencement date. The percent change shall be computed by comparing the base figure with the index figure published for the month, which begins each successive l2-month period. For example, a lease which commences in June of 1981 would have as the base figure the index published for May of 1981, and that figure would be compared with the index published for May of 1982, May of 1983, and so on, to determine the percent change. The U.S. Department of Labor Revised Consumer Price Index shall measure the cost of living index for Wage Earners and Clerical Workers, U.S. City Average, All Items figure, (1967=100) published by the Bureau of Labor Statistics. Payment shall be made with the monthly installment of fixed rent.

Rental adjustments shall be effective on the anniversary date of the lease. Payment of the adjusted rental rate shall become due on the first workday of the second month following the publication of the cost of living index for the month prior to the lease commencement date.

If Government exercises an option to extend the lease term at the same rate as that of the original term, the option price shall be based on the adjustments during the original term. Annual adjustments will continue.

In the event of any decreases in the cost of living index occurring during the term of occupancy under the lease, the rental amount will be reduced accordingly. The amount of any such reductions will be determined in the same manner as increases in rent provided under this clause.

The base for the operating costs adjustment will be $<insert base amount for operating costs>

D16 – Plans (8/02)

Thirty days after occupancy, Lessor must submit a final set of hard copy as-built plans and reproducible floor plans in l/8" scale depicting rented space and identifying entrances, exits, stairs, windows, partitions, closets, architectural, construction documents to include but not limited to electrical, mechanical, structural, fire protection, plumbing plans, architectural plans, lighting plans, furniture plans, installation plans, typical workstations, etc. A CAD program compatible with the latest release of AutoCAD and accessible and readable by the Government for future use shall generate the plans. The file(s) will be dwg format. All files shall be submitted on 3-1/2-inch double-sided, high-density diskettes, or, if approved by the RECO, on CD-ROM or QIC (1/4-inch cartridge) tape. The submitted disks shall be labeled with building name, address, list of drawing(s), date of the drawing(s), and Lessor's architect and phone number. The Lessor’s operators shall demonstrate the submission on FAA equipment, if requested by the RECO.

D17 – Progressive Occupancy (10/96)

The Government shall pay rent only when the entire premises or suitable units are ready for occupancy. If the agency occupies the space in partial increments, rent will accrue or be paid on a pro rata basis. Rental payments shall become due on the first workday of the month following the month in which an increment of space is occupied, except that should an increment of space be occupied after the fifteenth day of the month, the payment due date will be the first workday of the second month following the month in which it was occupied. The commencement date of the firm term will be a composite determined from all dates of incremental occupancy.

D18 – Recycling (1/12)

Where State or local law, code, or ordinance requires recycling programs (including those for mercury containing lamps) for the space to be provided, the Lessor shall comply with such State and/or local law, code, or ordinance, to the extent practicable, and provided that any such law, code, and/or ordinance is at least as stringent as the requirements set forth under Federal law and in the immediately following paragraph below.

In all other cases, the Lessor shall establish a recycling program for (at a minimum) paper, corrugated cardboard, glass, plastics, and metals where local markets for recovered materials exist. In addition, the Lessor shall provide an easily accessible, appropriately sized (2 square feet per 1,000 square feet of building gross floor area) area that serves the tenant space for the collection and storage of materials for recycling. Telecom rooms are not acceptable as recycling space. The Lessor agrees, upon request, to provide the Government with additional information concerning recycling programs maintained in the building and in the leased space after lease execution.

D19 – Recycled Content Products (Comprehensive Procurement Guidelines) (4/12)

A. To the extent feasible, the Lessor shall comply with Section 6002 of the Resource Conservation and Recovery Act of 1976 (RCRA). As required by this lease or in any succeeding lease entered into by and between the FAA and the Lessor, the Lessor shall use recycled content products as designated by the U.S. Environmental Protection Agency (EPA) in the Comprehensive Procurement Guidelines (CPG), 40 CFR Part 247, and its accompanying Recovered Materials Advisory Notice (RMAN). The CPG lists the designated recycled content products. EPA also provides recommended levels of recycled content for these products. The list of designated products, EPA's recommendations, and lists of manufacturers and suppliers of the products can be found at the www.epa.gov/cpg/products.htm web site.

B. The Lessor, if unable to comply with both the CPG and RMAN lists, shall submit to the RECO a request for waiver for each non-compliant material. The request for waiver shall be based on one of the following criteria: 1.) the cost of the recommended product is unreasonable; 2.) inadequate competition with respect to product manufacturers; 3.) compliant items are not available within a reasonable period of time; or 4.) items do not meet the requirements of Attachment A to this Lease.

D20 – Tax Adjustment (4/12)

A. The Government shall pay additional rent for its share of increases in real estate taxes over taxes paid for the calendar year in which its lease commences (base year). The estimated taxes for the base year are $<insert estimated taxes> for the building and $<insert estimated taxes> for the FAA’s leased premises.  The Lessor shall submit the real estate taxes for the base year and the current year or the anniversary date of the lease, whichever is later. Any payment by FAA in accordance with this paragraph shall be in a lump sum and become due after the execution of a supplemental lease agreement by the parties.  The Government will be responsible for payment only if the receipts are submitted within 60 calendar days of the date the tax payment is due. If no full tax assessment is made during the calendar year in which the Government lease commences, the base year will be the first year of a full assessment.

The amounts shown above for Base Year taxes shall be adjusted to the actual amount shown on Lessor’s tax bill (if the amount of the tax bill is different) and Lessor shall give the FAA written notice of the discrepancy, which notice shall include a copy of the tax bill for the Base Year.  A Supplemental Lease Agreement illustrating the corrected amount of the Base Year taxes will be signed by both parties.

B. The Government's share of the tax increase will be based on the ratio of the square footage occupied by the Government to the total rentable square feet in the building. If the Government's lease terminates before the end of a calendar year, payment will be based on the percentage of the year in which the Government occupied space. The payment will not include penalties for non-payment or delay in payment. If there is any variance between the assessed value of the Government's space and other space in the building, the Government may adjust the basis for determining its share of the tax increase.

C. The Government may contest the tax assessment by initiating legal proceedings on behalf of the Government and the Lessor or the Government alone. If the Government is precluded from taking legal action, the Lessor shall contest the assessment upon reasonable notice by the Government. Subject to the availability and adequacy of funds, the Government shall reimburse the Lessor for all costs the Lessor can prove are attributable to the contested tax, and shall execute all documents required for the legal proceedings. The Lessor shall agree with the accuracy of the documents. The Government shall receive its share of any tax refund. If the Government elects to contest the tax assessment, payment of the rent shall become due on the first workday of the month following conclusion of the appeal proceedings.

D. In the event of any decreases in real estate taxes occurring during the term of occupancy under the lease, the rental amount will be reduced accordingly. The amount of any such reductions will be determined in the same manner as increases in rent provided under this clause.

D21 – Time Extensions (10/96)

The lease will not be terminated nor the Lessor charged with resulting damage if delays arise from unforeseeable causes beyond the control of the Lessor and/or his contractors, subcontractors, suppliers, or another Government contractor. However, the Lessor shall notify the RECO, in writing, of any delay within 10 calendar days after it begins. The RECO shall ascertain the facts, determine the extent of the delay, and grant extensions when justified.

D22 – Unauthorized Negotiating (10/96)

In no event shall the Lessor enter into negotiations concerning the space leased or to be leased with anyone other than the RECO or his/her designee.

D23 – Wiring For Telephones (10/96)

The Government reserves the right to provide its own telephone service in the space to be leased. It may have inside wiring and telephone equipment installed by the local telephone company or a private contractor. Alternately, the FAA may wish to consider using inside wiring provided by the building, if available. However, the final decision will remain the Government’s.

V. SECTION E – SECURITY REQUIREMENTS

E1 – Facility Security (4/12)

Security requirements for Government occupied space must meet minimum-security accreditation standards for the type of facility covered by this lease. The FAA Facility Security Management Program defines facility security accreditation standard levels.  The security requirements identified below are tailored specifically for the type of facility covered by this lease. The Lessor shall provide or make accommodation to provide for all the security requirements listed herein for the leased premises covered by this lease agreement:

[Contact your local Servicing Security Element (SSE) at       for a list of your security requirements.  If there are none, then insert 'None' after the first paragraph]

The local SSE will determine any additional security upgrades that are required to meet accreditation and shall conduct a final security assessment of the building.  The Lessor shall provide maintenance services to the security upgrades installed by the Lessor within the leased premises and covered under this lease.

E2 – Contractor Personnel Suitability Requirements (10/13)

(a) This clause applies to the extent that this contract requires contractor employees, subcontractors, or consultants to have unescorted access to FAA:

(1) Facilities;

(2) Sensitive information; and/or;

(3) Resources regardless of the location where such access occurs, and none of the exceptions of FAA Order 1600.72A, Contractor and Industrial Security Program, Chapter 5, paragraphs 4, 6, 7 and 8 pertains.

Definitions of applicable terminology are contained in the corresponding guidance and FAA Order 1600.72A, appendix A.

(b) Consistent with FAA Order 1600.72A, the FAA Servicing Security Element (SSE) has approved designated risk levels for the positions under the contract. Those designated risk levels are:

<[To be entered by the RECO based on the OPM Position Designation Automated Tool or 1600-77(s) approved by the SSE]>

(c) If a National Agency Check with Inquiries (NACI) or other investigation is required under paragraph (b) for a given position, the Lessor will submit to the Contracting Officer (CO) a point of contact (POC) that will enter applicant data into the Vendor Applicant Process (VAP) system (vap.faa.gov). VAP is a FAA system used to process and manage security information for FAA contractor personnel. Each contract may have up to 5 POCs. Once designated, a VAP administrator will provide each POC a Web ID and password.

The type of investigation conducted will be determined by the position risk level designation for all duties, functions, and/or tasks performed and will serve as the basis for granting a favorable employment suitability authorization as described in FAA Order 1600.72A. If an employee has had a previous U. S. Government conducted background investigation which meets the requirements of Chapter 5 of FAA Order 1600.72A and Homeland Security Presidential Directive 12 (HSPD-12), it will be accepted by the FAA. However, the FAA reserves the right to conduct further investigations, if necessary. The contract may include positions that are temporary, seasonal, or under escort only. In such cases, an OPM Position Designation or FAA Form 1600-77 for each specific position will be established as the investigative requirements may differ from the NACI.

The following information must be entered into VAP by the POC for each applicant requiring an investigation:

- Name;

- Date and place of birth (city and state);

- Social Security Number (SSN);

- Position and office location;

- Contract number;

- Current e-mail address and telephone number (personal or work); and

- Any known information regarding current security clearance or previous investigations (e.g. the name of the investigating entity, type of background investigation conducted, contract number, labor category (Position), and approximate date the previous background investigation was completed).

If a prior investigation exists and there has not been a 2 year break in service by the applicant, the SSE will notify the Lessor that no investigation is required and that final suitability is approved.

If no previous investigation exists, the SSE will send the applicant an e-mail (this step may be delegated to VAP POC):

- Stating that no previous investigation exists and the applicant must complete a form through the Electronic Questionnaires for Investigations Processing (eQIP) system;

- Instructing the applicant how to enter and complete the eQIP form;

- Providing where to send/fax signature and release pages and other applicable forms; and

- Providing instructions regarding fingerprinting.

The applicant must complete the eQIP form and submit other required material within 15 days of receiving the e-mail from the SSE.

For items to be submitted outside eQIP, the Lessor must submit the required information, referencing the contract number, to:

Headquarters Contracts:

Manager, Personnel Security Division, AIN-400

800 Independence Avenue, S.W., Room 315

Washington, D.C. 20591

Regional and Center Contracts Contacts:

<[RECO insert appropriate Regional or Center information here or enter "none" if not applicable]>

(d) The Lessor must submit the information required by paragraph (c) of this Clause for any new employee not listed in the Lessor’s initial submission who is hired into any position identified in paragraph (b) of this Clause.

(e) The RECO will provide notice to the Lessor when any Lessor employee is found to be unsuitable or otherwise objectionable, or whose conduct appears contrary to the public interest, or inconsistent with the best interest of national security. The Lessor must take appropriate action, including the removal of such employee from working on this FAA contract, at their own expense. Once action has been taken, the Lessor will report the action to the CO and SSE.

(f) No Lessor employee will work in a high, moderate, or low risk position unless the SSE has received all forms necessary to conduct any required investigation and has authorized the Lessor employee to begin work.

(g) The Lessor must notify the RECO within one (1) business day after any employee identified pursuant to paragraph (c) of this Clause is terminated from performance on the contract. This notification must be done utilizing the Removal Entry Screen of VAP. If FAA issued the terminated employee an identification card, the Lessor must collect the card and submit it to the SSE.

(h) The Lessor must request a report from the VAP on at least a semiannual basis in order to reconcile discrepancies and then must notify the SSE of these discrepancies as soon as possible.

(i) The RECO may also, after coordination with the SSE and other security specialists, require Lessor employees to submit any other security information (including additional fingerprinting) deemed reasonably necessary to protect the interests of the FAA. In this event, the Lessor must provide, or cause each of its employees to provide, such security information to the SSE, to meet the requirements of paragraph (c) of this Clause.

(j) The Lessor and/or subcontractor(s) must contact the Servicing Security Elements (Regional and/or Center Security Divisions) or AIN-400 at Headquarters within one (1) business day in the event an employee is arrested (detained by law enforcement for any offenses, other than minor traffic offenses) or is involved in theft of government property or the Lessor becomes aware of any information that may raise a question about the suitability of a Lessor employee.

(k) Failure to submit information required by this clause within the time required may be determined by the RECO as a material breach of the contract.

(l) If subsequent to the effective date of this contract, the security classification or security requirements under this contract are changed by the Government and if the changes cause an increase or decrease in direct contract costs or otherwise affect any other term or condition of this contract, the contract will be subject to an equitable adjustment.

(m) The Lessor agrees to insert terms that conform substantially to the language of this clause, including paragraph (k) but excluding any reference to the Changes clause of this contract, in all subcontracts under this contract that involve access and where the exceptions under Chapter 5, FAA Order 1600.72A do not apply.

(n) Lessor employees who have not undergone a background investigation must be escorted at all times. In some instances, a Lessor employee may be required to serve as an escort. To serve as an escort, a Lessor employee must have a favorably adjudicated fingerprint check and initiated a NACI with FAA.

< Must be inserted into all SFOs and leases when the Lessor may require access to FAA facilities, sensitive information and/or resources. RECO is to insert information.>

Foreign Nationals as Contractor Employees (10/13)

(a) Each Lessor or subcontractor employee under this contract having access to FAA facilities, sensitive information, or resources must be a citizen of the United States of America, or an alien who has been lawfully admitted for permanent residence as evidenced by Alien Registration Receipt Card Form I-151, or who presents other evidence from the U.S. Citizenship and Immigration Service that employment must not affect his/her immigration status.

(b) Aliens and foreign nationals proposed under this contract must meet the following conditions in accordance with FAA Order 1600.72A, chapter 5, paragraph 7 & 8:

(1) Must have resided within the United States for three (3) of the last five (5) years unless a waiver of this requirement is requested and approved in accordance with the requirements stated in FAA Order 1600.72A, chapter 5, paragraph 9;

(2) A risk or sensitivity level designation can be made for the position; and

(3) The appropriate security-related background investigation/inquiry can be adequately conducted.

(c) Interim suitability requirements may not be applied unless the position is low/moderate in risk, and/or temporary, and/or is not in a critical area position.

Access to FAA Systems and Government-Issued Keys, Personal Identity Verification (PIV) Cards, and Vehicle Decals (10/13)

(a) It may become necessary for the Government to grant access to FAA systems or issue keys, PIV cards, vehicle decals, and/or access control cards to Lessor employees. Prior to or upon completion or termination of the work required hereunder, the Lessor must return all such Government-issued items and submit a request to terminate all user accounts on applicable FAA systems to the issuing office with notification to the Contracting Officer's Representative (COR). When Lessor employees who have been issued such items are terminated or no longer required to perform the work, the Government-issued items must be returned to the Government and a request submitted for the termination of FAA system access within three (3) business days after termination of the contract or the employee. Improper use, possession or alteration of FAA issued keys, PIV Cards and/or vehicle decals is subject to penalties under Title 18, USC 499, 506, 701, and 1030.

(b) In the event such keys, PIV Cards, or vehicle decals are lost, stolen, or not returned, the Lessor understands and agrees that the Government may, in addition to any other withholding provision of the contract, withhold [CO to enter appropriate amount] for each key, PIV Card, and vehicle decal lost, stolen, or not returned. If the keys, PIV Cards, or vehicle decals are not returned within 30 calendar days from the date the withholding action was initiated, any amount so withheld must be forfeited by the Lessor.

(c) Access to aircraft ramp/hangar areas is authorized only to those persons displaying a flight line identification card and for vehicles, a current ramp permit issued pursuant to Title 49, Part 1542, Code of Federal Regulations.

(d) The Government retains the right to inspect inventory, or audit PIV Cards, keys, vehicle decals, and access control cards issued to the Lessor in connection with the contract at the convenience of the Government. Any items not accounted for, to the satisfaction of the Government will be assumed to be lost and the provisions of section (b) apply.

(e) Keys must be obtained from the COR who will require the Lessor to sign a receipt for each key obtained. Lost or stolen keys, PIV Cards, vehicle decals, and access control cards must immediately be reported concurrently to the Contracting Officer (CO), COR, and [CO to insert name of local security division or staff and facility management office]. Electronic keying cards are handled in the same manner as metal keys.

(f) Each contract employee, during all times of on-site performance at the [CO to insert location] must prominently display his/her current and valid PIV card on the front portion of his/her body between the neck and waist. Each PIV card holder must not affix pins, stickers, or other decorations to the PIV.

(1) Prior to any Lessor employee obtaining a PIV Card or vehicle decals, the Lessor is required to enter data for each employee into the Vendor Applicant Process (VAP) as described in AMS clause 3.14-2, Contractor Personnel Suitability Requirements.  From the information entered into the VAP, the SSE will determine whether final suitability can be granted due to the existence of a previous investigation, or will initiate the Lessor applicant into the Electronic Questionnaires for Investigations Processing (eQIP) system so that the applicant can complete the investigative forms. Interim suitability cannot be granted until the eQIP form is completed, and fingerprints and signature pages are submitted to the SSE.  When an interim is granted by the SSE, the individual may begin work under escort until their OPM fingerprint check has been returned and successfully adjudicated. Once the OPM fingerprint check has been successfully adjudicated, they can then be badged. If the contract employee requires a PIV Card, a fingerprint check must be completed and favorably adjudicated by the SSE prior to approval or issuance of the PIV card.

(2) To obtain the PIV Card, Lessor employee must submit an identification Card/Credential Application (DOT 1681) signed by the Lessor employee and by the authorized trusted agent (when applicable) and also by the authorized sponsor to the CO or to the COR. The DOT 1681 must contain, as a minimum, under the "Credential Justification" heading, the name of the Lessor/company, the contract number or the appropriate acquisition identification number, the expiration date of the contract or the task (whichever is sooner), and the required signatures. The Lessor will be notified when the DOT 1681 has been approved and is ready for processing by the [CO to insert name and location of the person who will process the document]. Arrangements for processing the identification cards, including photographs and lamination can be made by the contacting [CO to insert point of contact with phone number].

(3) The Lessor must contact the SSE to obtain the procedures that the Lessor’s employees must utilize to obtain their PIV Card.

(g) The Lessor is responsible for ensuring final out-processing is accomplished for all departing Lessor employees. Final out-processing must be accomplished by close of business the final workday of the Lessor employee or the next day under special conditions. The SSE must be notified in writing and ensure that all FAA media, including the PIV card, are returned to the SSE.

VI. SECTION F - SPECIAL REQUIREMENTS

F1-SPECIAL POWER

F2-OTHER (SPECIFY)

F3-PAINTING FREQUENCY IN PUBLIC AREAS

F4-HVAC OVERTIME

VII. SECTION G - CERTIFICATION FORMS

ACM Survey Report (reference Asbestos Building Survey in III. Section C, Subparagraph C-12B for Buildings Construction prior to 1981)

ABAAS Compliance Report

Certification of Seismic Compliance

Lessor’s Annual Cost Statement

Safety and Environmental Certification Checklist

Vendor/Miscellaneous Payment Information Form